### SMA CONNECTORS

# HIGH PERFORMANCE SMA CONNECTORS

| ##### Electrical |  |
| --- | --- |
| Impedance | 50 Ohms Nominal |
| Frequency Range | DC – 18 GHz |
| Voltage Rating | 500 Volts Peak |
| D.W.V | 1000 Volts RMS |
| VSWR | 1.01x.005f (GHz) |
| Insertion Loss | .03x e f (GHz) dB |
| ##### Mechanical |  |
| Mating Torque | 15 inch pounds |
| Cable Attachment | Solder |
| Cable Retention | 60lbs |
| ##### Materials |  |
| Body | Gold Plated or Passivated Stainless Steel |
| Contact | Gold Plated Beryllium Copper |
| Insulator | PTFE |
| ##### Environmental |  |
| Temperature Range | -65°F to +165°F |
| Temperature Cycling | MIL-Std. 202 Method 102 (Test Condition C) |
| Moisture Resistance | MIL-Std. 202 Method 106 (Test Condition B) |
| Corrosion | MIL-Std. 202 Method 101 (Test Condition B) |
| Vibration | MIL-Std. 202 Method 204 (Test Condition D) |
| Altitude | MIL-Std. 202 Method 105 (Test Condition C) |

The SMA series of connectors from Coaxial Precision Assemblies are manufactured to meet the military interface standards defined in MIL-STD 348.  These series of connectors are tuned to provide an optimum match when installed on the cable assemblies.

These connectors are available from stock for both .141 and .085 semi rigid cables, including designs for low loss cables.

Please contact the factory or your local representative for price and delivery.

| PART NUMBER | CABLE TYPE |
| --- | --- |
| 2000-0101-02 | 0.141 semi-rigid |
| 2000-0102-02 | 0.141 LL semi-rigid |
| 2000-0103-02 | 0.085 semi-rigid |
| 2000-0104-02 | 0.085 LL semi-rigid |

SMA MALE FOR SEMI-RIGID CABLES

SMA FEMALE FOR SEMI-RIGID CABLES

| PART NUMBER | CABLE TYPE |
| --- | --- |
| 2001-0101-01 | 0.141 Semi-rigid |
| 2001-0102-01 | 0.141 LL semi-rigid |
| 2001-0103-01 | 0.085 semi-rigid |
| 2001-0104-01 | 0.085 LL semi-rigid |